DRAFT NBS Specification

Troldtekt® acoustic panels installed with concealed Troldtekt KN brackets on suspended C60 profile system.

(Suspended Ceiling System)

K40

135	SUSPENDED CEILING SYSTEM

- Please refer to Architect’s drawings ……………………. for locations

- Structural soffit: ………………………

- Ceiling system manufacturer/supplier: Troldtekt A/S, Sletvej 2A, 8310 Tranbjerg J, Denmark Tel: +45 8747 8100 Fax +45 8747 8111 Email: sales@troldtekt.co.uk

- Product reference: Troldtekt® acoustic

- Panel size: 600X600mm, 600X1200mm, 600X2400mm, special size.

- Thickness: 35mm

- Panel finish: FUTURECEM natural grey / Natural wood / Standard colours (White 101 / Grey 202 / Grey 208 / Black 207 / Clay 210) special colour

- Structure: Extreme fine 0.5mm, Ultrafine 1mm, Fine 1.5mm

- Edge design: K0-N - Square edges, K5-N - 5mm bevel, K11-N - 11mm bevel

- Ceiling type: Fine fibre wood wool panels meeting EN 13168 and EN 13964 standards. The panels must be available as PEFC (PEFC/09-31-030) - and FSC® (FSC® C115450)-certified.

- Certified “Gold” Cradle to Cradle” and the "Allergy Friendly Product Award" - "Allergy UK".

- Grid form: C60 Troldtekt MF channel sections

- Grid exposure: Concealed

- Suspension system: Troldtekt C60 system.

- Primary C60 grid profile centres: Maximum 1200mm

- Hanger centres: Maximum 1000mm

- C60 main runner centres: 600mm

- C-cross connectors at C60 primary grid/ C60 main runner intersections.

- Ceiling module: 600X600mm, 600X1200mm, 600X2400mm.

- Ceiling suspension depth: ………..(Variable)

- Manufacturer’s Troldtekt suspension components to include C60 Connectors, Adjustable C-suspension hangers, Suspension rods, C-cross connectors, Perimeter channel angle profile, Troldtekt KN brackets and Troldtekt screws.

- Insulation:	30 or 50mm stone wool (45kg/m3) acoustic pads (If required to complete the required Absorber Class ceiling system)

325 INSTALLING ADJUSTABLE SUSPENSION HANGERS

- Installation: Install Troldtekt Adjustable C-suspension hangers vertically and securely to the structural soffit using appropriate fasteners to be sufficiently long to provide adequate fix.

- Spacing: Maximum or as C60 MF Troldtekt grid manufacturer’s requirements.

335 PERIMETER CHANNEL TRIM

- Jointing: Neat and accurate, without lipping or twisting

- External and internal corners: Mitre joints

Intermediate butt joints: Minimize. Use longest available lengths of trim. Align adjacent lengths.

- Fixing: Fix firmly to perimeter wall or other building structure with appropriate fixings.

375 INSTALLING PANEL CEILING SYSTEM

- Installation should be in accordance with manufacturer’s instructions, see www.troldtekt.co.uk  for download.

- Moisture content of boards at time of fixing: Appropriate to end use.

- It is important that the Troldtekt panels are allowed to acclimatize prior to installation. Acclimatization means that the panels must adapt to the temperature and humidity in the room where they are to be installed.

- Troldtekt panels to grid installed in broken bond

- Cutting: Cut panels neatly and accurately in accordance with manufacturer’s instructions.

- Fixing: Start by installing a complete row of Troldtekt panels along the centre line of the room and work with rows of panels from the centre. It is important that the panels are installed closely together, and the optimum aesthetic effect is obtained by installing the panels in broken bond. Screw the first line of brackets into the C60 profile along the centre line of the room using two screws per bracket. Use two self-tapping screws (4 × 13mm) per bracket.

- Install the first bracket against the joint. Then lead the groove of the panel over the bracket and install the next line of bracket continuously as you install the panels. It is important that the panels are installed tightly together.

- Fix the Troldtekt panels along the wall, lengthwise, using self-tapping Troldtekt screws, (4.2 × 55mm / 4.2 x 50mm) 25/20mm from the edges. The maximum distance between the screws should be 600mm. Be careful not to overdrive the screws and so cause an indentation in the panel.

- Place 30 or 50mm stone wool acoustic pads over Troldtekt panels and fit tightly between the C60 grid sections, cutting when necessary.

- Movement joints: To coincide with any movement joints in the surrounding structure.

- Exposed board edges are to be site painted with Troldtekt Touch-up Paint.

395 INTEGRATED SERVICES

- General: Position services accurately, support adequately. Align and level in relation to the ceiling and suspension system. Do not diminish performance of ceiling system.

- Small fittings: Support with rigid backing boards or other suitable means. Do not damage or distort the ceiling.

- Surface spread of flame rating of additional supporting material: Not less than ceiling material.

- Services outlets:

- Supported by ceiling system: Provide additional hangers.

- Independently supported: Provide flanges to support or trim ceiling system.