DRAFT NBS Specification

Troldtekt® acoustic panels for wall cladding installed with Troldtekt screws on wooden battens

(Wall lining)

K11

- WALL LINING

Please refer to Architect’s drawings ……………………. for locations

Substrate: Wooden battens (minimum size    x    mm)

- Additional supports: ………………………

- Wall lining manufacturer/supplier: Troldtekt A/S, Sletvej 2A, 8310 Tranbjerg J, Denmark Tel: +45 8747 8100 Fax +45 8747 8111 Email: sales@troldtekt.co.uk

- Product reference: Troldtekt® acoustic

- Panel size: 600X600mm, 600X1200mm, 600X2400mm, special size.

- Thickness: 25mm, 35mm

- Panel finish: FUTURECEM natural grey / Natural wood / Standard colours (White 101 / Grey 202 / Grey 208 / Black 207 / Clay 210) special colour

- Structure: Extreme fine 0.5mm, Ultrafine 1mm, Fine 1.5mm

- Edge design: K0 - Square edges, K0-U - Square edges with U-trace, K5 - 5mm bevel, K5-F - With bevel, K5-FU - Shiplap with rebate for U-groove, K11 - 11mm bevel.

- Lining type: Fine fibre wood wool panels meeting EN 13168 and EN 13964 standards. The panels must be available as PEFC (PEFC/09-31-030) - and FSC® (FSC® C115450)-certified.

- Certified “Gold” Cradle to Cradle” and the "Allergy Friendly Product Award" - "Allergy UK".

- Lining module: 600X600mm, 600X1200mm, 600X2400mm.

- Access:All panel units are screw fastened and demountable

- Manufacturer’s Troldtekt components to include Troldtekt screws for wood.

- Insulation:	30 or 50mm stone wool (45kg/m3) acoustic pads.

Setting out

- Long edges horizontal and centred on supports.

Fixing to supports

- Fasteners: Troldtekt wood screw 4.2 x 45mm (25mm panels), 4.2 x 55mm (35mm panels). Installation should be in accordance with manufacturer’s instructions, see www.troldtekt.co.uk  for download.

- Fixing: Install battens (minimum size   x    mm) horizontally with a centre spacing of 600mm starting from the base of the wall. Set out the Troldtekt panels along the wall base starting from the centre of the wall, ensuring that the end cut panels are of equal size. The optimum aesthetic effect is obtained by installing the panels in broken bond.

- Position and install the first line of Troldtekt panels along the base of the wall, ensuring that the panels are installed closely together. Fix the panels using Troldtekt screws spaced at maximum 600mm centres positioned at 25mm from the panel edges. Be careful not to overdrive the screws and so cause an indentation in the panel.

- If required place 30 or 50mm stone wool acoustic pads behind Troldtekt panels and fit tightly between battens fixing where necessary.

940 BOARD MOISTURE CONTENT AND CONDITIONING

- Moisture content of boards at time of fixing: Appropriate to end use.

- It is important that the Troldtekt panels are allowed to acclimatize prior to installation. Acclimatization means that the panels must adapt to the temperature and humidity in the room where they are to be installed.